# Valve flow from Cv

Core theory and limitations

This worksheet estimates gas flow through a valve from its flow coefficient Cv and the upstream and downstream states. Cv describes valve capacity: numerically, it is the US gallons per minute of water near 60°F that produce a 1 psi valve pressure drop. Gas sizing additionally accounts for expansion and choking.

# The correlation used here

q=22.67CvP1(1−2x3)xGgT1for x < 0.5

Here q is SCFM, P₁ is upstream psia, T₁ is upstream degrees Rankine, Gg is gas molar mass divided by air molar mass, and x = (P₁−P₂)/P₁. The constant 22.67 is specific to these units. The square-root term represents pressure-driven flow; the factor 1−2x/3 accounts approximately for gas expansion.

For x ≥ 0.5 the sheet uses q = 0.471 × 22.67 × Cv × P₁/√(GgT₁). Thus, flow becomes independent of further decreases in downstream pressure. A larger Cv or upstream pressure raises capacity; a heavier gas or higher absolute temperature reduces the predicted standard volume flow.

# Reference flow and property choices

The underlying gas correlation reports SCFM at 14.7 psia and 60°F. Density at that reference state converts standard volume to mass flow through ṁ = ρref q, with consistent units. Actual volume then follows from mass flow divided by upstream density. The separate Nm³/h output uses its selected reference pressure and temperature.

Calculated and package density choices affect these mass and volume conversions. They do not add real-gas compressibility, a valve-specific choking parameter or additional correction factors to the underlying Cv gas-flow equation. All pressure ratios use absolute pressure; Rankine equals °F + 459.67.

# Assumptions and limitations

This is a simplified legacy gas-valve correlation. Its fixed choking threshold x = 0.5 is not universal: actual valve choking depends on trim geometry, pressure recovery and gas heat-capacity ratio. Current ISA/IEC methods use valve-specific factors such as xT, together with expansion, piping and gas-property corrections.

Use a Cv appropriate to the valve opening and flow direction. The model assumes steady single-phase gas flow and does not capture condensation, two-phase behavior, changing actuator position, connected piping losses or valve noise and vibration limits. For equipment selection, compare with the manufacturer’s sizing method and coefficients.

# References

Emerson, Control Valve Handbook, Chapter 5 — valve sizing, expansion factors and valve-specific choking.

ISA, ISA75 valve standards — the worksheet identifies legacy S75.01 and S75.02 methods; consult the applicable current sizing and test standards.