# Polytropic pipe flow

Core theory and limitations

This worksheet estimates steady compressible flow through a pipe with wall friction, fittings and elevation change. Falling pressure expands a gas, increasing its velocity and altering the loss. Kirkland’s polytropic approximation represents that expansion without solving the full gas-dynamics equations.

# Friction and available pressure

K=fLD+ΣKi;ΔP−ρgΔz=Kρu22Y2

K combines pipe and fitting resistance, f is the Darcy friction factor, L/D is length divided by inside diameter, u and ρ are inlet velocity and density, Δz is elevation rise, and Y is the net expansion factor. Fittings use equivalent lengths or direct loss coefficients. Reynolds number Re = ρuD/μ governs f: 64/Re in laminar flow and a roughness-dependent Swamee–Jain correlation in turbulent flow.

Mass continuity gives ṁ = ρuA. Because the friction factor depends on flow, the mass rate is adjusted until the modeled loss matches the available pressure. The elevation term uses inlet density.

# Expansion and choking

Pρc=constant;c=1+1.8x(k−1)K+1

The polytropic exponent c approximates the pressure–density path; x is the fractional pressure drop ΔP/P₁ and k = cₚ/cᵥ. The fitted exponent and expansion factor approximate subsonic Fanno flow: adiabatic flow of a perfect gas with friction. The alternatives set c = 1 for isothermal behavior or c = k for an isentropic pressure–density relation; friction remains irreversible.

A fitted critical pressure ratio limits the pressure drop used in the model. Beyond this point, further lowering downstream pressure does not increase predicted mass flow. This pipe-friction choking criterion differs from the ideal short-nozzle criterion.

# Assumptions and limitations

The model is steady, one-dimensional and based on a constant-area, calorically perfect gas approximation. The choking fit is intended for K ≥ 1 and 1 < k ≤ 1.67; transitional flow needs particular care. Property-package density or viscosity does not turn the polytropic equations into a general real-gas solver.

Strong property variation, phase change, substantial heat exchange and complex local fitting flow can invalidate the approximation. Elevation is a lumped correction. Check the pressure residual, regime and validity indicators before interpreting a calculated flow.

# References

W. M. Kirkland (2019), A Polytropic Approximation of Compressible Flow in Pipes With Friction, J. Fluids Eng. 141, 121404; Eqs. 25 and 34.

Repository reference, Pressure drop in pipe fittings and valves — equivalent length and resistance coefficient; fitting-loss background.